SPECIFICATIONS: FOLDING ARM AWNING HAROL LX530

Harol or equivalent

1. General

- The Harol LX530 is a folding arm awning box of the three-tube type.
- The construction is characterised by a torque bar to which the arms, the supports and the roller tube are attached. 
- A standard base plate and an optional rear plate fully seal the entire awning.
- The colour of the aluminium parts and profiles: RAL colour of choice from the official RAL range.

2. The profiles

2.1. The torque bar

- Material: a square welded tube 40 x 40 mm, manufactured from a zinc-plated zendzimir 2-mm strip.

- Colour: RAL colour of choice from the official RAL range.  
- Function: the other components are attached to this supporting base profile

2.2.  The fabric roll

- - Material: a round welded tube manufactured from a zinc-plated zendzimir strip

- - Function: for rolling the fabric up and down.

- -  Provided with a groove for attaching the fabric.

- -  Diameter: Ø85 x 1.25 mm.

2.3. The Clip-On Clip-Off profile

- Material: extruded PVC clip profile.
- Function: attachment of the fabric to the fabric roll without the use of screws
- Allows fast and easy assembly and disassembly of the fabric.

2.4. The cover profile

Material: extruded aluminium with powder-coating.

Function: covers the fabric over the whole width, along the top.

Provided on the inside with a plastic sliding profile, for protection and to allow a smooth guiding of the fabric.

Fitted with a rubber seal on the back (not possible if a back cover is fitted).

2.5. The extension pole

- -  Material: extruded aluminium with powder-coating.

- -  Function: to close off the awning at the front.

- -  Provided with:

two grooves at the top for the attachment of the fabric and cloth of the fabric split covering. 
at the bottom, a groove for the attachment of a valance (optional).

- At the ends finished with aluminium covers: 
-  coated in the colour of the extension pole 
-  fully hides the arms.

-  Optional:  Variovolant: see point 10.2

2.6. The bottom plate

- Material: extruded aluminium with powder-coating.

- Colour: standard in the chosen frame colour (see colour chart.)

- Function: covers the bottom of the awning over the whole width.

- Is suspended underneath the square torque bar by means of plastic snap-on brackets.

3. The folding arms

- Connect the box and the extension pole.
- Available with the following drops: 1500, 2000, 2500, 3000, 3500 and 4000mm.
- Up to a width of 6000mm two arms are mounted, from 6001mm up to 7000mm 3 arms are mounted.

- The arm profiles:
- 	Material: ëxtruded aluminium with powder-coating.

- The hinges:
-  Material: made of a cast aluminium alloy.
-  Hinges on durable sliding bearings of sintered bronze, with a PTFE sliding layer.

- The springs and the steel cables:
Material: pre zinc-plated, high-quality steel.
The cable diameter is always in relation to the force applied.

- The power of both springs is divided over two steel cables: 
- deduplication of the forces.
- considerable extension of the operating life.

3.1. The mounting brackets

- To fix the screen to the base surface required, a number of special fastening supports are available:

- Wall supports: (standard).

- Is used for normal assembly against a wall.

- Material: extruded aluminium with powder-coating.

- Width: 100 mm and provided with three fastening holes for M12.

- At the front provided with an opening to slide the torque bar in

- The wall supports should be placed as close as possible near the folding arms.

- The number of wall brackets supplied always depends on the screen width.

- Additional supports available (optional).

- In the event of an unstable foundation, it is recommended to mount additional wall brackets, in order to divide the weight of the awning over several points.

| Awning width | Number of supports |
| --- | --- |
| 2000 - 3500mm | 2 |
| 3501 - 6000mm | 3 |
| 6001 - 7000mm | 4 |

4. Maximum dimensions and weights:

- Width:
- Maximum:

- - Minimum:

- - 6m in 1 section with 2 arms.
- 7m in 1 section with 3 arms.
- 12m in 2 sections with 4 arms.
- Depending on the chosen drop
- Awnings with 2 arms: W min = drop + 570mm
- Awnings with 3 arms: B min = (drop x 1.5) + 760 mm - For the linked awnings: see point 5.

- - Drop: Maximum: 4.0m - Minimum: 1.5m

- - The approximate weight: Approx. 19 kg per metre (width)

- Operation

- - One operating system: - Electric operation:

- - Electric operation: - Standard:

- - motor with automatic fine-tuning, the awning will always close completely (SLT). - 3 metres of electric cable.
- single-pole, surface-mounted switch and plug.

- - For group controls and automation, see the Harol Home Automation File. Note: The top and bottom position of the motor come with factory settings.

6. Fabrics

- - Acrylic:
- Composition: Mass-coloured acrylic fibres.
- Colour: choose from the Dickson Orchestra collection (other collections optional)

- Weight: approx. 290 - 300 gr/m2.
- Thickness: approx. 0.5 mm
- Manufacture:

- - vertical strips of 1200 mm.
- side hems of 50 mm.
- double step stitch with polyester thread.

- - The fabrics are treated for rot, are mould-proof and dirt-repellent.

7. Optional

- Variovolant in the extension pole

- Integrated LED lighting in the box, wireless operation with RTS transmitter

- Controls:

- Group control for cabled motors:

  - using a relay box

Wind protection by means of a Wind (and/or) solar device: when wind is detected, folding arm is automatically closed, when specified lux value is detected, folding arm automatically slides open.

Cables for power supply and operation/control of motors not included.

- Group control for wireless motors: central operation for multi-channel transmitter

Wind protection by means of a Wind (and/or) solar device: when wind is detected, folding arm is automatically closed, when specified lux value is detected, folding arm automatically slides open, however, wireless communication between transmitter and motor